K SANDHIYA(192521378)

1. Real-Time Interrupt Scheduling Mechanism

Objective: Assign interrupt priorities according to sampling rate, urgency, and processing requirements, while avoiding excessive CPU overhead.

ALGORITHM RealTimeInterruptScheduler

INPUT:

Sensors = {S1, S2, ..., Sn}

For each sensor Si:

sampling_rate[Si]

urgency[Si]

processing_time[Si]

deadline[Si]

interrupt_pending[Si]

BEGIN

FOR each sensor Si DO

// Calculate priority score

rate_score = Normalize(sampling_rate[Si])

urgency_score = Normalize(urgency[Si])

processing_score = Normalize(processing_time[Si])

deadline_score = CalculateDeadlineUrgency(deadline[Si])

priority[Si] =

W1 * rate_score

+ W2 * urgency_score

+ W3 * deadline_score

+ W4 * processing_score

IF priority[Si] > MAX_PRIORITY THEN

priority[Si] = MAX_PRIORITY

END IF

END FOR

WHILE system_is_running DO

IF interrupt_request_received THEN

Add requesting sensors to InterruptQueu

// Sort highest priority first

Sort InterruptQueue by:

1. Priority (highest first)

2. Earliest deadline

3. Waiting tim

WHILE InterruptQueue is not empty DO

S = RemoveHighestPriority(Interrup

Disable lower-priority interrupts tem

ServiceInterrupt(S

ReadSensorDat

StoreDataInBuffer(S)

IF ProcessingRequired(S) THEN

ScheduleProcessing(S)

END IF

ClearInterruptFlag(S)

RestoreInterruptState()

END WHILE

END IF

// Avoid unnecessary polling

CPU enters low-power/idle state

until next interrupt or timer event

END WHILE

END

Key points

- Sensors with higher sampling rates and greater urgency receive higher priority.

- Deadline-based scheduling ensures time-critical data is handled first.

- Interrupts are queued when several sensors request service simultaneously.

- CPU overhead is reduced by using interrupt-driven operation instead of continuous polling.

- Lower-priority processing can be postponed when a critical interrupt arrives.

2. Double-Buffered DMA Mechanism

Objective: Continuously transfer data between an I/O device and memory using two buffers so that one buffer can be processed while the other receives new data.

ALGORITHM DoubleBufferedDMA

INPUT:

BufferA

BufferB

DMA_Controller

IO_Device

BEGIN

Initialize BufferA

Initialize BufferB

active_buffer = BufferA

processing_buffer = BufferB

Configure DMA_Controller with active_buffer

Start DMA transfer from IO_Device

to active_buffer

WHILE system_is_running DO

IF DMA_Transfer_Complete THEN

Mark active_buffer as FULL

Generate DMA_COMPLETE_INTERRUPT

// Switch buffers

temp = active_buffer

active_buffer = processing_buffer

processing_buffer = temp

IF active_buffer is EMPTY THEN

Configure DMA_Controller

with active_buffer

Restart DMA transfer

ELSE

// Both buffers are full

Generate BUFFER_OVERFLOW_WARNING

Stop or temporarily throttle input

HandleOverflow()

END IF

// Process previously completed buffer

IF processing_buffer is FULL THEN

ProcessData(processing_buffer)

Clear processing_buffer

Mark processing_buffer as EMPTY

END IF

END IF

IF BUFFER_OVERFLOW_DETECTED THEN

Generate OVERFLOW_INTERRUPT

Reduce input rate if possible

Increase processing priority

IF possible THEN

Increase buffer size

END IF

IF data loss is unavoidable THEN

RecordLostData()

END IF

END IF

END WHILE

END

Working principle

I/O Device

|

v

+---------+       +---------+

| Buffer A| <---> | Buffer B|

+---------+       +---------+

|                 |

v                 v

DMA Transfer     CPU Processing

While Buffer A is being filled by DMA, the CPU processes Buffer B. After Buffer A is full, DMA switches to Buffer B and the CPU processes Buffer A.

This minimizes data loss and allows continuous data acquisition.

3. I/O Scheduling Algorithm

Objective: Select the next I/O request by considering priority, request size, waiting time, latency, and available bandwidth while preventing starvation.

ALGORITHM IOScheduler

INPUT:

PendingRequests = {R1, R2, ..., Rn}

For each request Ri:

device_priority[Ri]

request_size[Ri]

waiting_time[Ri]

latency_requirement[Ri]

required_bandwidth[Ri]

BEGIN

WHILE PendingRequests is not empty DO

FOR each request Ri in PendingRequests DO

priority_score =

W1 * Normalize(device_priority[Ri])

+ W2 * Normalize(waiting_time[Ri])

+ W3 * Normalize(1 / latency_requirement[Ri])

+ W4 * Normalize(required_bandwidth[Ri])

- W5 * Normalize(request_size[Ri])

// Prevent starvation

IF waiting_time[Ri] > STARVATION_LIMIT THEN

priority_score =

priority_score + STARVATION_BOOST

END IF

IF latency_requirement[Ri] is CRITICAL THEN

priority_score =

priority_score + DEADLINE_BOOST

END IF

score[Ri] = priority_score

END FOR

// Select request with highest score

R = RequestWithMaximumScore(PendingRequests)

// Check available bandwidth

IF AvailableBandwidth >= required_bandwidth[R] THEN

Remove R from PendingRequests

StartIOTransfer(R)

Wait until transfer completes

UpdateDeviceStatistics(R)

ELSE

// Bandwidth insufficient

Find another request that can be serviced

IF another suitable request exists THEN

R = SelectNextRequest()

StartIOTransfer(R)

ELSE

WaitForBandwidth()

END IF

END IF

// Update waiting times

FOR each remaining request Ri DO

waiting_time[Ri] =

waiting_time[Ri] + elapsed_time

END FOR

END WHILE

END

Important features

- Device priority handles important devices first.

- Waiting time increases priority over time.

- Latency requirements give urgent requests faster service.

- Request size and bandwidth are considered for efficient transfers.

- Starvation prevention is achieved using aging/starvation boosting.

4. DMA Channel Arbitration Mechanism

Objective: Decide which DMA device gets memory-bus access when multiple devices request it simultaneously.

ALGORITHM DMAChannelArbitration

INPUT:

DMA_Requests = {D1, D2, ..., Dn}

For each request Di:

priority[Di]

transfer_size[Di]

urgency[Di]

waiting_time[Di]

deadline[Di]

BEGIN

WHILE DMA_Requests are pending DO

FOR each Di in DMA_Requests DO

urgency_score =

CalculateUrgency(deadline[Di])

fairness_score =

Normalize(waiting_time[Di])

size_score =

CalculateTransferEfficiency(transfer_size[Di])

arbitration_score =

W1 * priority[Di]

+ W2 * urgency_score

+ W3 * fairness_score

+ W4 * size_score

// Prevent starvation

IF waiting_time[Di] > MAX_WAIT_TIME THEN

arbitration_score =

arbitration_score + FAIRNESS_BOOST

END IF

score[Di] = arbitration_score

END FOR

// Select DMA request

D = RequestWithMaximumScore(DMA_Requests)

GrantMemoryBus(D)

StartDMATransfer(D)

Transfer data for:

MIN(transfer_size[D], MAX_BURST_SIZE)

ReleaseMemoryBus(D)

Update waiting times of other requests

IF transfer of D is complete THEN

Remove D from DMA_Requests

ELSE

Keep D in DMA_Requests

END IF

IF HighPriorityDeadlineIsNear() THEN

IncreasePriorityOfUrgentRequests()

END IF

END WHILE

END

Explanation

The arbitration mechanism uses a weighted priority score:

Arbitration Score =

Priority

+ Urgency

+ Waiting Time

+ Transfer Efficiency

A high-priority and urgent DMA request gets the bus quickly, while the waiting-time factor prevents low-priority devices from starving.

The use of a maximum burst size also prevents one DMA device from monopolizing the memory bus.

5. Adaptive I/O Management Mechanism

Objective: Dynamically change communication parameters when congestion, delays, or errors occur while maintaining reliable communication with low CPU overhead.

ALGORITHM AdaptiveIOManager

INPUT:

Devices = {D1, D2, ..., Dn}

For each device Di:

communication_mode[Di]

transfer_size[Di]

baud_rate_or_speed[Di]

error_count[Di]

latency[Di]

throughput[Di]

retry_count[Di]

BEGIN

Initialize communication parameters

WHILE system_is_running DO

FOR each device Di DO

// Monitor device

status = CheckTransferStatus(Di)

Measure:

latency[Di]

throughput[Di]

error_count[Di]

queue_length[Di]

IF status == ERROR THEN

error_count[Di]++

IF retry_count[Di] < MAX_RETRIES THEN

retry_count[Di]++

RetryTransfer(Di)

ELSE

// Communication quality is poor

ReduceTransferSize(Di)

ReduceCommunicationSpeed(Di)

IF AlternativeModeAvailable(Di) THEN

SwitchCommunicationMode(Di)

END IF

ResetRetryCount(Di)

END IF

ELSE IF latency[Di] > LATENCY_LIMIT THEN

ReduceTransferSize(Di)

IncreaseDevicePriority(Di)

AllocateMoreCPU/DMAResources(Di)

ELSE IF queue_length[Di] > CONGESTION_LIMIT THEN

// Congestion detected

ReduceInputRate(Di)

EnableDMAIfAvailable(Di)

IncreaseBufferSizeIfPossible(Di)

ELSE IF throughput[Di] < MIN_THROUGHPUT THEN

AdjustTransferParameters(Di)

IF AlternativeCommunicationModeAvailable(Di) THEN

SelectMoreEfficientMode(Di)

END IF

ELSE

// Normal operation

MaintainCurrentParameters(Di)

END IF

// Recover performance when conditions improve

IF PerformanceImproved(Di) THEN

GraduallyIncreaseTransferSize(Di)

GraduallyIncreaseCommunicationSpeed(Di)

IF BetterModeAvailable(Di) THEN

SwitchToBetterMode(Di)

END IF

END IF

END FOR

// Avoid continuous CPU polling

SleepUntil:

TimerEvent OR

DMAInterrupt OR

IOInterrupt OR

ErrorEvent

END WHILE

END

Main features

- Performance monitoring: Tracks throughput, latency, errors, and queue length.

- Congestion detection: Reduces input/transfer rate when buffers or queues become overloaded.

- Error handling: Retries failed transfers and changes communication parameters after repeated failures.

- Dynamic resource allocation: Gives additional DMA/CPU resources to devices experiencing critical delays.

- Adaptive transfer size: Uses smaller transfers during congestion and larger transfers when conditions improve.

- Low CPU overhead: Uses interrupts, DMA, and timer-based monitoring rather than continuously polling every device.

- Reliability: Retransmission and fallback communication modes help prevent data loss.