K SANDHIYA

(192521378)

1. I/O Subsystem for a Real-Time Robotic Arm Controller

Requirements: Position sensors produce data every 500 μs (2 kHz), while control commands must be processed within 5 μs.

Proposed architecture

Position Sensors

│

▼

Sensor Interface ──► Priority Interrupt Controller

│                         │

│                    High-priority ISR

│                         ▼

│                  Real-Time Controller

│                         │

│                    Motor Actuators

│

└──────────────► DMA Buffer

│

▼

Diagnostic Memory

Operation

- Position sensors are connected to a dedicated sensor interface.

- Each critical sensor generates a high-priority interrupt when new data arrives.

- The interrupt controller uses priority-based interrupt handling, ensuring critical sensor interrupts preempt lower-priority activities.

- The ISR should be extremely short: capture the sensor value, timestamp it, and place it in a real-time buffer.

- Control commands are processed through the highest-priority real-time task/ISR so that the 5 μs deadline is met.

- Non-critical diagnostic information is transferred through DMA, avoiding CPU involvement for every data item.

- Double buffering or ring buffers can prevent data loss while the CPU processes one buffer and DMA fills another.

Result: Critical sensor/control operations receive deterministic CPU service, while DMA handles background diagnostic I/O efficiently.

2. Communication Architecture for an Autonomous Vehicle

The cameras generate a combined 2 GB/s, equivalent to approximately 16 Gb/s, so a high-bandwidth interconnect is required.

Proposed architecture

Cameras

┌────┬────┬────┐

▼    ▼    ▼    ▼

Camera Interfaces

│

▼

PCIe Switch

│

┌──────┴──────┐

▼             ▼

High-Speed GPU   Main Memory

│             │

└──────┬──────┘

▼

Perception System

Safety Sensors ──► Real-Time Bus/MCU

│

▼

Priority Interrupt Controller

│

▼

Safety Controller ──► Actuators

Low-Priority Devices ──► Separate/Lower-Priority I/O Path

Design

- High-bandwidth cameras connect through PCIe, using multiple PCIe lanes as required by the aggregate bandwidth.

- Camera data is transferred directly into memory/GPU memory using DMA, minimizing CPU copying.

- Safety-critical sensors use a dedicated deterministic communication path, such as a real-time bus or dedicated MCU.

- Safety interrupts receive the highest priority.

- PCIe traffic from cameras and other high-volume devices is isolated from safety-critical traffic using separate controllers, queues, or PCIe resources.

- Low-priority devices use lower-priority queues and cannot monopolize CPU or bus resources.

- Buffering and flow control prevent camera bursts from delaying safety messages.

Result: Camera data receives high-throughput PCIe communication, while safety-critical sensor communication remains deterministic and isolated from low-priority traffic.

3. I/O Mechanism for an Industrial Monitoring System

There are 100 continuously operating sensors, so generating one CPU interrupt for every measurement could create excessive processor overhead.

Proposed architecture

100 Sensors

│

▼

Sensor Interface / I/O Controller

│

├──────────────► Critical Fault Detector

│                       │

│                Immediate Interrupt

│                       ▼

│                    CPU/ISR

│

▼

Sensor FIFO / Buffer

│

▼

Interrupt Coalescing

│

▼

CPU

Operation

- Sensor measurements are first collected by an I/O controller or FIFO.

- Interrupt coalescing is used for normal sensor data.

- For example, the controller may interrupt the CPU after a specified number of measurements or after a specified timeout.

- The CPU then processes multiple sensor readings during one interrupt instead of servicing 100 separate interrupts.

- A critical fault signal bypasses interrupt coalescing.

- Fault signals immediately generate a high-priority interrupt.

- The fault ISR identifies the affected sensor and initiates the required protective action.

- Ring buffers/FIFOs ensure that measurements are not lost between interrupt events.

Result: Normal measurements cause fewer CPU interrupts, while critical faults receive immediate service.

4. High-Speed Network Interface Subsystem

The network may reach 10 Gb/s, so processing every packet through a CPU interrupt would create significant overhead.

Proposed architecture

10-Gb/s Network

│

▼

┌──────────────┐

│ Network NIC  │

└──────┬───────┘

│

│ DMA

▼

┌──────────────┐

│ Ring Buffers │

│  in Memory   │

└──────┬───────┘

│

▼

Packet Processing

│

▼

Interrupt after

configurable batch

Design

- Use a 10-Gb/s network interface card (NIC) with DMA support.

- Incoming packets are transferred directly from the NIC into preallocated memory buffers.

- The CPU does not need to copy every packet from the NIC.

- Descriptor-based ring buffers are used for efficient packet management.

- Instead of generating an interrupt for every packet, the NIC generates an interrupt after a configurable batch, for example:

  - every N packets, or

  - after a specified timeout.

- Interrupt moderation should be configurable so the system can trade latency against CPU overhead.

- Multiple receive queues can be assigned to different CPU cores to support parallel processing.

- For extremely high traffic, hardware packet filtering can discard irrelevant packets before they reach the CPU.

Result: DMA and interrupt batching significantly reduce CPU overhead while maintaining high throughput at 10 Gb/s.

5. I/O Architecture for a Smart Healthcare Monitoring System

Different patient-monitoring devices produce data at different rates, while emergency signals require immediate attention.

Proposed architecture

Patient Devices

┌────────┬────────┬────────┬───────────┐

│ ECG    │ SpO₂   │ Temp.  │ Emergency │

│ High   │ Medium │ Low    │ Signal    │

└───┬────┴───┬────┴───┬────┴─────┬─────┘

│        │        │           │

▼        ▼        ▼           ▼

DMA      DMA      DMA      Highest-Priority

Channels  Channels  Channel     Interrupt

│        │        │           │

└────────┴────────┴─────┐      ▼

│  Emergency ISR

▼

Main Memory

│

▼

Monitoring System

Design

- Each monitoring device is assigned an appropriate I/O channel according to its sampling rate.

- Continuous monitoring data is transferred using DMA, allowing devices to write directly into memory without requiring an interrupt for every sample.

- Circular/ring buffers can be used for continuous ECG, temperature, oxygen saturation, and other measurements.

- Emergency devices generate highest-priority interrupts.

- A nested interrupt controller allows an emergency interrupt to preempt a lower-priority monitoring operation.

- The emergency ISR should immediately perform essential actions such as recording the event, notifying the control system, or activating an alarm.

- Normal DMA transfers continue in the background without unnecessarily interrupting critical processing.

- DMA buffers should be protected from overwriting unread data.

Result: Continuous patient data is efficiently stored using DMA, while emergency events receive immediate, highest-priority CPU attention.

Summary

| System | Main I/O Technique | Critical Requirement |
| --- | --- | --- |
| Robotic arm | Priority interrupts + DMA | Control response within 5 μs |
| Autonomous vehicle | PCIe + DMA + isolated safety path | 2 GB/s camera data without affecting safety |
| Industrial monitoring | Interrupt coalescing + immediate fault interrupts | 100 sensors with fast fault detection |
| 10-Gb/s network | DMA + packet batching | Reduce CPU packet-processing overhead |
| Healthcare monitoring | DMA + nested interrupts | Emergency signals get highest priority |

Overall, the designs use interrupts for time-critical events and DMA for high-volume/non-critical data, which minimizes CPU overhead while maintaining deterministic response where required.